Rev. Dated: 3/1/2022

Clark

SPEC NUMBER: 02-C25CRP-DESECTCRP25#-1

STOCK CODE:  C25CRP

BAG NAME:  Desect Crop Insecticide

BAG WEIGHT: 25 lb, 11.34 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  20” (w) x 23-1/2” (l) x 6-1/2” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 60NK/60NK/50BEX

CLOSING FEATURES: N/A

PERFORATIONS:  ½” x 1/2” Micro Perfs Inner Two Plies, Micro Perfs Outer Ply

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  6-1/2”, Upper Right

- TYPE:  Tuck In Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.   2,000 Bags per Pallet

PRINT

- PER ARTWORK: AW-02-C25CRP-DESECTCRP25#-010121

- COLOR DISTRIBUTION: Process Yellow, Process Magenta, Process Cyan, Process Black, Green (PMS 336)

- INK COVERAGE: 40%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices