Rev. Dated: 3/17/2022

Clark

SPEC NUMBER:  02-C30DST-DESECT30#-1

STOCK CODE: C30DST

BAG NAME:  Desect

BAG WEIGHT:  30 lb, 13.61 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  20” (w) x 23-1/2” (l) x 6-1/2” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 2/60NK, 50BEX

CLOSING FEATURES: N/A

PERFORATIONS: 1/2”x1/2” Inner 2 Plies, Micro Perfs Outer Ply

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  5-1/2 Reduced”, Upper Right

- TYPE:  Tuck In Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

- Packaged for Outside Storage, Double Stacked

B.   2,000 Bags per Pallet

PRINT

- PER ARTWORK: AW-02-C30DST-DESECT30#-070119

- COLOR DISTRIBUTION:  5 Color. Process Black/Cyan/Magenta/Yellow,

Green (PMS 336U)

- INK COVERAGE: 50%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices