Rev. Dated: 3/14/2022

Clark

SPEC NUMBER: 02-C458-CELATOM20KG-1

STOCK CODE: C458

BAG NAME:  Celatom

BAG WEIGHT: 20 kg, 44.09 lb

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  22” (w) x 35” (l) x 7” (tb)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION:  3/60NK

CLOSING FEATURES: N/A

PERFORATIONS: 1/2”x1/2” Inner 2 Plies, 1”x1” Outer Ply

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  6” Reduced, Upper Right

- TYPE:  Poly Internal

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.   1500 Bags per Pallet

C.

PRINT

- PER ARTWORK:  AW-02-C458-CELATOM20KG-070121

- COLOR DISTRIBUTION: 1 Color. Orange (PMS 159)

- INK COVERAGE: 30%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices