Rev. Dated: 2/7/2022

Clark

SPEC NUMBER:  02-C506-CELATOM50#-1

STOCK CODE:  C506

BAG NAME:  Celatom

BAG WEIGHT:  50 lb, 22.68 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  23” (w) x 33-1/2” (l) x 7” (tb)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION:  3/60NK

CLOSING FEATURES: N/A

PERFORATIONS:  ½” x ½” Inner Two Plies, 1” x 1” Outer Ply

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  6” Reduced, Upper Right

- TYPE:  Poly Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.  1,400 Bags per Pallet

C.

PRINT

- PER ARTWORK:  AW-02-C506-CELATOM50#-080121

- COLOR DISTRIBUTION: 1 Color. Orange (PMS 151/1505)

- INK COVERAGE:  40%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices