Rev. Dated: 3/14/2022

Clark

SPEC NUMBER: 02-C511-NATURALDEAFA50#-1

STOCK CODE: C511

BAG NAME: Natural DE Animal Feed Additive

BAG WEIGHT: 50 lb, 22.68 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  23” (w) x 35” (l) x 7” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 2/50NK, 50BEX

CLOSING FEATURES: N/A

PERFORATIONS:  1/2”x1/2” Inner 2 Plies, 1”x1” Outer Ply

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  6” Reduced, Upper Right

- TYPE:  Poly Internal

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.   1500Bags per Pallet

PRINT

- PER ARTWORK: AW-02-C511-NATURALDEAFA50#-040121

- COLOR DISTRIBUTION: 3 Color. Brown (PMS 499), Green (PMS 576),

Green (PMS 5773)

- INK COVERAGE:  60%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices