Rev. Dated 03/25/2025

Lovelock

SPEC NUMBER:  03-911N-PLAINBROWN50#-1

STOCK CODE: 911N

BAG NAME: Plain Brown

BAG WEIGHT: 50 lb, 22.68 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  23-1/2” (w) x 35” (l) x 7” (tb)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION:  55HPK/50HPK

CLOSING FEATURES: N/A

PERFORATIONS:  Micro Perfs All Plies

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  5-1/2 Reduced”, Upper Right

- TYPE:  Tuck-In Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.  2000 Bags Per Pallet

PRINT

- PER ARTWORK:  AW-03-911N-PLAINBROWN50#-080121

- COLOR DISTRIBUTION: 3 Color. Red (PMS 185), White (PMS 7014), Black (PMS6014)

- INK COVERAGE: 25%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

OTHER REQUIREMENTS

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices.