Rev. Dated: 3/16/2022

Fernley

SPEC NUMBER: 07-B1001-PLAINDE33#-1

STOCK CODE: B1001

BAG NAME:  Plain DE

BAG WEIGHT: 33#, 15.12KG

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  20” (w) x 23” (l) x 5” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼”)

CONSTRUCTION:  50HPK/55HPK

CLOSING FEATURES: N/A

PERFORATIONS: 1/2”x1/2” Inner Ply, 1’x1” Perfs Outer Ply

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  5”, Upper Right

- TYPE:  Poly Internal

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.   3000 Bags per Pallet

C.

PRINT

- PER ARTWORK: AW-07-B1001-PLAINDE33#-040122

- COLOR DISTRIBUTION:  1 Color. Black

- INK COVERAGE: 15%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date