Rev. Dated: 223/2022

Jackson, MS

SPEC NUMBER: 85-2400-NOVASILAF25KG-1

STOCK CODE:  2400

BAG NAME: 	Novasil AF

BAG WEIGHT:  25 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  18” (w) x 21” (l) x 4-1/2” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION:  52HPK/.5mil PE/52HPK/60BEX

CLOSING FEATURES: N/A

PERFORATIONS:  N/A

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  4-1/2”, Upper Left

- TYPE:  Poly Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Inside Storage, Double Stacked

B.   3000 Bags per Pallet

C.

PRINT

- PER ARTWORK: AW-85-2400-NOVASILAF25KG-090117

- COLOR DISTRIBUTION: 1 Color. Blue (PMS P300)

- INK COVERAGE: 25%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices