Rev. Dated: 3/14/2022

Clark

SPEC NUMBER:  85-4736-CLAYCATALYST50#-1

STOCK CODE: 4736

BAG NAME: Clay Catalyst

BAG WEIGHT:  50 lb, 22.68 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  20” (w) x 25” (l) x 4” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 50NK/40NK/1.0 mil HDPE/40NK/40NK/50NK

CLOSING FEATURES: N/A

PERFORATIONS:  1” x 2” All plies except Outer ply

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  4”, Upper Left

- TYPE:  Poly Internal

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Inside Storage, Double Stacked

B.   1500 Bags per Pallet

PRINT

- PER ARTWORK:  AW-85-4736-CLAYCATALYST50#-050118

- COLOR DISTRIBUTION:  3 Color. Black (PMS 3046), Red (PMS 187),

White (PMS 3400)

- INK COVERAGE:  25%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices