Rev. Dated: 2/23/2022

Jackson, MS

SPEC NUMBER:  85-8253-NOVASILPLUS25KG-1

STOCK CODE: 8253

BAG NAME:  Novasil Plus

BAG WEIGHT: 25kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  18” (w) x 21” (l) x 4-1/2” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 52HPK/.5 mil PE/52HPK/60BEX

CLOSING FEATURES: N/A

PERFORATIONS:  N/A

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  4-1/2”, Upper Left

- TYPE:  Poly Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Inside Storage, Double Stacked

B.   3,000 Bags per Pallet

C.

PRINT

- PER ARTWORK: AW-85-8253-NOVASILPLUS25KG-090117

- COLOR DISTRIBUTION: 1 Color. Black

- INK COVERAGE: 25%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices