Rev. Dated: 2/23/2022

Jackson, MS

SPEC NUMBER: 85-8176-NOVASILANTICAKE25KG-1

STOCK CODE: 8176

BAG NAME: Novasil Anti-Cake

BAG WEIGHT: 25 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  18” (w) x 21” (l) x 4-1/2” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼”)

CONSTRUCTION:  52HPK/.5 mil PE/52HPK/60BEX

CLOSING FEATURES: N/A

PERFORATIONS:  N/A

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  4-1/2”, Upper Left

- TYPE:  Poly Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Inside Storage, Double Stacked

B.   3000 Bags per Pallet

C.

PRINT

- PER ARTWORK:  AW-85-8176-NOVASILANTICAKE25GK-100117

- COLOR DISTRIBUTION: 4 Color. Blue (PMS P300), Red (PMS 186), Yellow (PMS 012), Black

- INK COVERAGE: 25%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date