Rev. Dated: 3/8/2022

Multiple Plant (Lovelock, Vale)

SPEC NUMBER: MP-300-SPBSWIMPOOL25#-1

STOCK CODE: 300

BAG NAME: SPB Swim Pool Filter

BAG WEIGHT: 25 lb, 11.33 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  20” (w) x 23-1/2” (l) x 5” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 60NK/60BEX

CLOSING FEATURES: N/A

PERFORATIONS:  Micro Perfs All Plies

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  5”, Lower Left

- TYPE:  Poly Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.   2,500 Bags per Pallet

C.

PRINT

- PER ARTWORK: AW-MP-300-SPBSWIMPOOL25#-050118

- COLOR DISTRIBUTION: 3 Color. Blue (PMS 541), Teal (PMS 319), Black (PMS 6014)

- INK COVERAGE: 50%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date