Rev. Dated: 06-23-2024

Multiple Plant (Vale, Lovelock)

SPEC NUMBER:  MP-811-CELATOM20KG-1

STOCK CODE:  811

BAG NAME:  Celatom White

BAG WEIGHT:  20 kg, 44.09 lb

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  22” (w) x 34” (l) x5” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION:  50HPK/55BEX

CLOSING FEATURES: N/A

PERFORATIONS:  Micro Perfs All Plies

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  5”, Upper Right

- TYPE:  Poly Sleeve

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A. Packaged for Outside Storage, Double Stacked

B.  2000 Bags per Pallet

C.

PRINT

- PER ARTWORK:   AW-MP-811-CELATOM20KG-082121

- COLOR DISTRIBUTION:  3 Color. Red (PMS 185), Purple (PMS 2623), Black (PMS 6014)

- INK COVERAGE:

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date