Rev. Dated: 3/25/2025

Multiple Plant Clark/Lovelock

SPEC NUMBER:  MP-817-DIACEL20KG-1

STOCK CODE: 817

BAG NAME:  Diacel Perdomini

BAG WEIGHT: 20 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  23” (w) x 35” (l) x 5” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 50HPK/55HPK

CLOSING FEATURES: N/A

PERFORATIONS:  Micro Perfs All Plies

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  5”, Upper Right

- TYPE:  Poly Internal

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.   2000 Bags per Pallet

PRINT

- PER ARTWORK: AW-MP-817-DIACEL20KG-080117

- COLOR DISTRIBUTION:  6 Color. Red (PMS 199), Reflex Blue,

Black (PMS 6014)

- INK COVERAGE: 35%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

=== OTHER REQUIREMENTS

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices.