Rev. Dated: 3/25/2025

Multiple Plant (Vale, Lovelock)

SPEC NUMBER: MP-916-ALAR50#-1

STOCK CODE: 916

BAG NAME: Alar F.A. 140-F

BAG WEIGHT:  50 lb, 22.68 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  23” (w) x 35” (l) x 5” (tbv)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 50HPK/55HPK

CLOSING FEATURES: N/A

PERFORATIONS:  Micro Perfs All Plies

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  5”, Upper Right

- TYPE:  Poly Internal

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.   2,000 Bags per Pallet

PRINT

- PER ARTWORK:  AW-MP-916-ALAR50#-040121

- COLOR DISTRIBUTION:  5 Color. Black (PMS 6014), Red (PMS 485), Blue (PMS 306), Yellow (PMS 107), White (PMS 7014)

- INK COVERAGE: 40%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

OTHER REQUIREMENTS

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices.