Rev. Dated: 3/25/2025

Multiple Plant (Clark, Lovelock)

SPEC NUMBER:  MP-C5016-WATERSOLUBLE50#-1

STOCK CODE: C5016

BAG NAME: Domestic Water-Soluble Powder

BAG WEIGHT:  50 lb, 22.68 kg

BAG STYLE:  Multi-Wall Paper, Pasted Valve, INCLUSIVE BAG

DIMENSIONS:  23” (w) x 35” (l) x 7” (tb)

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 3/BKU (Bleached Re-Pulpable)

CLOSING FEATURES: N/A

PERFORATIONS:1/2”x1/2” Inner 2 Plies, 1”x1” Outer Ply

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  6” Reduced, Upper Right

- TYPE:  Poly Internal

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Double Stacked

B.   2000 Bags per Pallet

PRINT

- PER ARTWORK: AW-MP-C5016-WATERSOLUBLE50#-050118

- COLOR DISTRIBUTION: 1 Color. Blue (PMS 2747)

- INK COVERAGE: 10%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

OTHER REQUIREMENTS

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices.