Rev. Dated: 03/25/2025

Multiple Plant (Lovelock, Blair)

SPEC NUMBER:  MP-P30-CELATOMPERLITE3.0CUFT-1

STOCK CODE:  P30

BAG NAME:  Perlite Filter Aid

BAG VOLUME:  3.0 cu ft

BAG STYLE:  Multi-Wall Paper, Pasted Valve

DIMENSIONS:  23” (w) x 36” (l) x  6-1/2” tbv

(Std. tolerances: width and top & bottom  1/4”, length  ¼ ”)

CONSTRUCTION: 55HPK/50HPK

CLOSING FEATURES: N/A

PERFORATIONS:  ½” x ½” Inner Ply, Micro Perfs Outer

VALVE: (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  6-1/2”, Upper Right

- TYPE:  Poly Internal

SPECIAL FEATURES:

- Heavy Duty Anti-Skid to 32-36 deg

- Top and Bottom to Front

PACKAGING REQUIREMENTS:

A.  Packaged for Outside Storage, Stacked Two High

B.  2,000 Bags per Pallet

PRINT

- PER ARTWORK: AW-MP-P30-CELATOMPERLITE3.0CUFT-080121

- COLOR DISTRIBUTION: 3 Color. Blue (PMS 2925), Green (PMS 3405), Black

- INK COVERAGE:  30%

- IDENTIFICATION ON BACK OF BAG:

P.O. Number

Mfg. Date

OTHER REQUIREMENTS

Packaging and packaging materials must comply with all FDA and European legislation requirements for food contact materials and be manufactured in accordance with relevant good manufacturing practices.