Rev. Dated: 26May16

PACIFIC

SPEC NUMBER:  06-MUS15-8

STOCK CODE:  90555

BAG NAME:  Min-U-Sil 15 Micron

BAG WEIGHT:  50#

BAG STYLE:  Pasted Valve Stepped End (PVSE))

DIMENSIONS:  18” (w) x 19” (l) x 6.5” (tb)

(Std. tolerances: width and top & bottom  1/8”, length  ¼ ”)

CONSTRUCTION:  2/50 Nat MWK, 1/50 Bleached MWK

CLOSING FEATURES:  N/A

PERFORATIONS:  Inner 2 plies, staggered micro perfs

VALVE:    (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  5-1/2” URHV

- TYPE:  Full Poly-Lok Insert

- MATERIAL:  1.0 mil HDPE w/33# NK liner board

- INTERNAL EXTENSION (from edge of ear):  n/a

- SLIT LENGTH: n/a

SPECIAL FEATURES:

A.  Heavy anti-skid to 33-36 deg.

B.  Top & bottom to face

C.  Two (2) sample bags from each order to be sent to plant purchasing department.

PRINT

- PER ARTWORK:  AW-06-MUS15-031715

- COLOR DISTRIBUTION:  2S2C   PMS Black, Orange (PMS 151U)

- INK COVERAGE:  20%

- IDENTIFICATION ON BACK OF BAG  (PER THE FOLLOWING FORMAT)

FREIGHT CLASSIFICATION STAMP

SPEC #                                 PO #

STOCK CODE                     MFG DATE