Rev Dated: 22Jun16

BERKELEY SPRINGS

SPEC NUMBER:  15-MUS40-4

STOCK CODE:  156836

BAG NAME:  Min-U-Sil 40 Micron

BAG WEIGHT:  50#

BAG STYLE:  Pasted Valve, Stepped End (PVSE)

DIMENSIONS:  17-1/2” (w) x 20-1/2” (l) x 3-1/2” (tbv)

(Std. tolerances: width and top & bottom  1/8”, length  ¼”)

CONSTRUCTION:  2/50 Natural MWK, 1/50 Bleached MWK

CLOSING FEATURES:  N/A

PERFORATIONS:  Inner 2 plies, staggered, 1/8” pins, 1”x1” centers

VALVE:    (Std. tolerances:  1/8”)

- SIZE & PLACEMENT:  3-1/2”, URHV

- TYPE:  PE Tubular Reinforced Insert

- MATERIAL:  1.0 mil, HDPE w/26# NK liner board

- INTERNAL EXTENSION (from edge of ear):  1-1/2”

- SLIT LENGTH:

SPECIAL FEATURES:

A.  Medium anti-skid to 28-32 deg.

B.  Top & bottom to face

- Bags must be flat and wrinkle free for auto packer

- Two (2) sample bags from each order to be sent to plant purchasing department

PRINT:

- PER ARTWORK:   AW-15-MUS40-031915

- COLOR DISTRIBUTION:  2S2C   PMS Black, Orange (PMS 151U)

- INK COVERAGE:  8.8%

- IDENTIFICATION ON BACK OF BAG  (PER THE FOLLOWING FORMAT)

FREIGHT CLASSIFICATION STAMP

SPEC #                                 PO #

STOCK CODE                     MFG DATE